HodoScope PMT High Voltage Cable and Signal Cable Prep Guide

- Signal Cable- two per PMT

  - Cut each cable to 18” length from the exit of connector housing

  - Cut 18” of Flame Retardant Wrap; feed signal cable through wrap

  - Strip 0.6” of outer insulation from cut end using cable stripper(16 Gauge)

  - Separate and twist copper braid to form separate stranded cable; remove foil lining from center conductor

  - Ready to solder to PCB

- High Voltage Cable- 1 per PMT

  - Cut each cable to 19” length

  - Feed nut, washer, and gasket onto cable, per Pasternack assembly guide http://www.pasternack.com/images/ProductPDF/PE44093.pdf

  - Cut .45” of insulation

  - Feed Clamp, and form braid over clamp to secure components

  - Cut and feed 18” of Flame Retardant wrap onto cable

  - Strip 0.6” off base end of cable, form braid into separate strand

  - Assemble bulk head nut, gasket, and lock washer

  - Place bulkhead, insulator, and contact in zip lock bag for solder assembly